- Go to this website and download the “Self Extracting Web Installer” for the latest version available. (This will prompt you to create an account & a name and address verification)

- Run the downloaded file

- This will take you to a welcome screen, click “Next”. Then it takes you to a sign-in screen. Enter your information and select “Download and Install Now”, then click “Next”.

- This will take you to a “Select Product to Install” screen. Make sure you ONLY select “Vivado”. Click “Next”.

- This will take you to a “Select Edition to Install” screen. Select “Vivado ML Standard”. Click “Next”.

- This will take you to a selection page. Match your selections with the image provided:

- This will take you to a “License Agreements” page. Select “I Agree” for all, then click “Next”.

- This will take you to a “Select Destination Directory” page. Enter your desired directory and then click next. (Most will just leave the default selection). Click “Next”

- This will take you to an “Installation Summary” page. Click “Install”

Testing Vivado

- Open Vivado (should be an icon on your desktop). Should take you to a screen like this:

- Select “Create Project” under Quick Start, click next, and name your project “hello_world”. Should look like this:

- Choose RTL Project (Don’t Specify Souces):

- Leave the next page as default, and select next, then finish:

- Add Your Hello World SystemVerilog File

  - In the Flow Navigator (left panel), click Add Sources.

  - Choose Add or Create Simulation Sources.

  - Click Create File.

  - File type: SystemVerilog

  - File name: hello.sv

  - Click OK.

  - Select the new file in the list and click Finish.

- Double click hello.sv in Sources

- This will open the hello.sv file, then paste the following code in:

module hello;

initial begin

$display("Hello, World!");

$finish;

end

endmodule

- In Flow Navigator (left panel), under Simulation, click Run Simulation → Run Behavioral Simulation.

  - If you get an error message saying “C:/Users/ngulv/OneDrive/Desktop/ECE231/ hello_world/hello_world.cache/wt' already exists, is a directory, but is not writable.” - Create a new folder called ECE231 in another place without OneDrive syncing and follow the instructions again.

- In the TCL Console, you should see “Hello, World!”