# Final Review 2025

Everything from tests 1 and 2

Complete the truth table

| 00/R 01/G 10 /B 11 x=0 x=1 x=0_ x=1 | Q1 | Q0 | X | D1 | D0 | R | G | B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 |  |  |  |  |  |
|  | 0 | 0 | 1 |  |  |  |  |  |
|  | 0 | 1 | 0 |  |  |  |  |  |
|  | 0 | 1 | 1 |  |  |  |  |  |
|  | 1 | 0 | 0 |  |  |  |  |  |
|  | 1 | 0 | 1 |  |  |  |  |  |
|  | 1 | 1 | 0 |  |  |  |  |  |
|  | 1 | 1 | 1 |  |  |  |  |  |

Complete the truth table

| 00/ 01/R 10 /BG 11 /RG x=0 x=1 x=0_ x=1 | Q1 | Q0 | X | D1 | D0 | R | G | B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 |  |  |  |  |  |
|  | 0 | 0 | 1 |  |  |  |  |  |
|  | 0 | 1 | 0 |  |  |  |  |  |
|  | 0 | 1 | 1 |  |  |  |  |  |
|  | 1 | 0 | 0 |  |  |  |  |  |
|  | 1 | 0 | 1 |  |  |  |  |  |
|  | 1 | 1 | 0 |  |  |  |  |  |
|  | 1 | 1 | 1 |  |  |  |  |  |

Given the truth table, fill in the state diagram
For any state that turns on a function, write after the slash.

Write every state transition showing which variable and which value is required to transition. X and Y are digital inputs.

| 00 / 01 / 10 / 11 / | Q1 | Q0 | X | Y | D1 | D0 | R | G | B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | X | 1 | 0 |  |  |  |
|  | 0 | 0 | 1 | X | 0 | 1 |  |  |  |
|  | 0 | 1 | X | 0 | 0 | 1 | 1 | 1 |  |
|  | 0 | 1 | X | 1 | 1 | 1 | 1 | 1 |  |
|  | 1 | 0 | X | 0 | 1 | 0 |  | 1 | 1 |
|  | 1 | 0 | X | 1 | 0 | 0 |  | 1 | 1 |
|  | 1 | 1 |  |  |  |  |  |  |  |
|  | 1 | 1 |  |  |  |  |  |  |  |

module test_values;

logic [7:0] a = 8'hA2, b = 8'h59, c = 8'hF2, d;

logic signed [7:0] e = 52;

logic [7:0] hamlet;

initial begin

d = c + 17;

$display("d = %d", d);

d = d - 4;

$display("d = %d", d);

e = -e;

$display("e = %b", e);

e = 8'b10001100;

e = -e;

$display("e = %b", e);

$display("e = %d", e);

c = c ^ a;

$display("c = %b", c);

a = a << 3;

$display("a = %b", a);

b = b >> 1;

$display("b = %b", b);

a = 8'hA2;

a = {a[4:0],a[7:5]};

$display("a = %b", a);

b = 8'h56;

b = (b << 1) | (b >> 7);

$display("b = %b", b);

c = 8'b01000100;

c = c * 4;

$display("c = %b", c);

$finish;

end

endmodule

/* reminder: 2s complement
01001100  ⇒ first invert every bit 10110011 then add 1 10110100

Given a 32-bit number x, write the expression to rotate it right by k
(x >> k) | ( x << (32-k))

rotating left by k

(x << k) | ( x >> (32-k))
for a 32-bit number rotating left by 5 is the same as rotating right by _____32-5=27
*/

Error Detection/Correction

Odd Parity: For each byte Identify whether there is an error or not by writing ERROR or CORRECT under each group

10011010_1   11001110_0  10111111_1  10000000_0

Given 9 bits, is it possible to tell whether the error is in the data or the parity bit itself (yes or no)? __________

Even Parity: For each byte Identify whether there is an error or not by writing ERROR or CORRECT under each group

10011010_0   11001110_0  10111111_1  10000000_0
CORRECT      ERROR        CORRECT   ERROR

Checksum: For each group identify if the data has arrived successfully

8’h41 8’h65 8’h41  checksum E7_____

Forward Error Correction (triplicate)

Given the message: 10100010 we send 111_000_110_001_000_011_111_000
Write the message received at the other end. _______________

How many error bits causes an error? _____________


1000 Base-T Ethernet transmits data at __________ Mbps
If it has 4 wire pairs, how much data is transmitted on each pair _____________

It is full duplex, what is the speed of transmission in each direction ___________Mbps

If there were 4 levels of signal, state the baud rate of each pair _______________Mbaud